README.DOC: Differences Between MCS and PCA-II Software

Contents:

MCS Batch Processing .................................... 1

MCS File Formats......................................... 2

MCS and Windows 386 ..................................... 3

New Command Line Switches................................ 4

Keyspeed Utility Program ................................ 5

PrintA Utility Program .................................. 6

Packing List ............................................ 7

MCS Switch Settings ..................................... 8

1. MCS Batch Processing.

(A) New Command:

DAC <mode>

where mode is equal to: SAW = sawtooth

MSB = MSB

TRI = triangle

(B) Changed Command:

MCS <passes> <time> <units>

where passes :the number of preset passes

time   :muliplier for time range: [1..65534] or 'e' (external)

(if units are MICRO min multiplier is 2)

units  :time units MICRO (microseconds)

MILLI (milliseconds)

SECO  (seconds)

MIN   (minutes)

HR    (hours)

DAY   (days)

(C) Deleted Commands:  (that were in PCA-II)

DATA

GAIN

OFFSET

SEARCH

TIME

2. MCS File Formats.

The following 'C' language source code is provided to illustrate

the exact format of the MCS spectrum file header and the MCS configuration

file.

#define DTLEN 13

#define MAXID 72

/*--------------------- MCS DATA TYPES ----------------------------- */

/* ---------------  Standard Data Types -------------- */

typedef unsigned char     BOOL;

typedef int               SHORT;

typedef unsigned int      USHORT;

typedef long int          LONG;

typedef unsigned long int ULONG;

typedef char              CHAR;

typedef unsigned char     UCHAR;

typedef float             FLOAT;

typedef double            DOUBLE;

/* --------------  Standard Data Types --------------- */

/* -------------- CALIB TYPEDEF ----------------------------------------------- */

typedef struct

{

CHAR   units[4];    /* units strings (ie: Kev, feet, gallons ...) */

DOUBLE calx0;       /* coefficients for: 0th order term */

DOUBLE calx1;       /* coefficients for: first order term  */

DOUBLE calx2;       /* coefficients for: second order term  */

SHORT  numbpoints;  /* number of calibration points */

DOUBLE calctrd[5];  /* centroids for up to 5 points */

DOUBLE calvalue[5]; /* values for up to 5 calibration points */

} CALIB;

/* -------------- CALIB TYPEDEF ----------------------------------------------- */

/* ---------------------- HEADER STRUCT ---------------------- */

typedef struct

{

UCHAR  etime[3];             /* (0 - 2)     */

UCHAR  ltimeflag;            /* (3  ) UNUSED */

UCHAR  rtimeflag;            /* (4  ) UNUSED */

UCHAR  convergain;           /* (5  ) UNUSED */

UCHAR  digoffset;            /* (6  ) UNUSED */

UCHAR  idcodestr[15];        /* (7 - 21)    file name root */

UCHAR  pca_date[12];         /* (22 - 33)   mmm dd yyyy    */

UCHAR  pca_time[12];         /* (34 - 45)   hh:mm:ss PM    */

UCHAR  group;                /* (46 )       */

UCHAR  units;                /* (47 )       system calibration flag - see control.units */

CALIB  calib;                /* (48 - 157)  110 byte calibration structure */

UCHAR  expansion[99];        /* (158 - 256) UNUSED */

UCHAR  phamode;              /* (257) UNUSED */

UCHAR  mcs;                  /* (258) UNUSED */

UCHAR  mcstimelab;           /* (259) UNUSED */

UCHAR  mcsdwellnumb;         /* (260) (not used by MCS product) */

UCHAR  mcspasct[3];          /* (261 - 263) UNUSED

UCHAR  centupflag;           /* (264)       */

UCHAR  fwhmupflag;           /* (265)       */

SHORT  numbchanspm;          /* (266 - 267) */

UCHAR  asday;                /* (268) [10c] acquisition start day (byte)        */

UCHAR  asmonth;              /* (269) [10d] acquisition start month (byte)      */

SHORT  asyear;               /* (270) [10e] acquisition start year (word)       */

UCHAR  ashund;               /* (272) [110] acquisition start hundredth  (byte) */

UCHAR  assec;                /* (273) [111] acquisition start sec (byte)        */

UCHAR  asmin;                /* (274) [112] acquisition start minute (byte)     */

UCHAR  ashour;               /* (275) [113] acquisition start hour (byte)       */

UCHAR  astpday;              /* (276) [114] acquisition stop day (byte)         */

UCHAR  astpmonth;            /* (277) [115] acquisition stop month (byte)       */

SHORT  astpyear;             /* (278) [116] acquisition stop year (word)        */

UCHAR  astphund;             /* (280) [118] acquisition stop hundredth (byte)   */

UCHAR  astpsec;              /* (281) [119] acquisition stop sec (byte)         */

UCHAR  astpmin;              /* (282) [11a] acquisition stop minute (byte)      */

UCHAR  astphour;             /* (283) [11b] acquisition stop hour (byte)        */

UCHAR  id[MAXID];            /* (284 - 355) ID string (72 byte null terminated ascii string) */

UCHAR  majvers;              /* (356) program major version number (byte) */

UCHAR  minvers;              /* (357) program minor version number (byte) */

ULONG  real_elap_time;       /* (358 - 361) (4 byte long int) */

UCHAR  acqstop_time[DTLEN];  /* (362 - 374) acquire stop time  hh:mm:ss PM  (12 byte ascii string) */

UCHAR  acqstop_date[DTLEN];  /* (375 - 387) acquire stop date  mmm dd yyyy (12 byte ascii string) */

UCHAR  acqstart_time[DTLEN]; /* (388 - 400) acquire start time  hh:mm:ss PM  (12 byte ascii string) */

UCHAR  acqstart_date[DTLEN]; /* (400 - 413) acquire start date  mmm dd yyyy (12 byte ascii string) */

UCHAR  mcs_id[4];            /* (414 - 417) MCS id string: = { 'M' 'C' 'S' '\0' } */

ULONG  preset_time;          /* (418 - 421 preset time in seconds */

UCHAR  dac;                  /* (422) MSB, TRI or SAW */

UCHAR  dwell_units;          /* (423) 'e' 'u' 'm' 'S' 'M' 'H' or 'D'  */

USHORT dwell;                /* (424-425) MCS dwell muliplier (1..65534) */

UCHAR  future[84];           /* (426 - 509) for future expansion  */

SHORT  endheader;            /* (510-  511) header end marker = 100 * version number (word) */

} HEADER;

/* ---------------------------------------- HEADER STRUCT ---------------------- */

/* ----------------------- CONFIG STRUCT ---------------------------------------- */

typedef struct

{

SHORT   vscale;            /* Log == (0), 256 == (1), 512 == (1), ... 16M == (17) */

SHORT   expand;            /* an index into the expand level: 0 -> 8192, 1 -> 4096, .. 6 -> 256 */

SHORT   group;             /* group number: 1 -> 63 (ie: control.group) */

UCHAR   pattern[65];       /* DIR (file_box) search pattern  */

UCHAR   filename[65];      /* active spectrum file           */

UCHAR   loadname[65];      /* file name to load              */

UCHAR   report_name1[80];  /* ascii out file name            */

UCHAR   report_name2[80];  /* ROI out file name              */

UCHAR   report_name3[80];  /* integrals out file name        */

UCHAR   peak_rpt_name[80]; /* file name for peak report      */

UCHAR   pcxname[80];       /* file name for PCX file name    */

UCHAR   batchname[80];     /* batch file name                */

UCHAR   username[80];      /* user program name              */

SHORT   dwell;             /* dwell time for MCS mode        */

UCHAR   dwell_units;       /* 'u' 'm' 's' 'M' 'H' 'D' or 'e' */

UCHAR   dac;               /* MSB, TRI or SAW                */

USHORT  initport;          /* I/O address of the MCS card    */

} CONFIG;

/* ---------------------- CONFIG STRUCT -------------------------------------------- */

3. MCS and Windows 386:

MCS software was not witten to be a 'Windows' application.

MCS and Windows 386 do conflict with each other in one known

area,  graphics.  This  is  because  the  assembly  language

graphics  functions in  MCS software  use the  EGA (or  VGA)

hardware registers (for speed).

Two conditions must occur for the conflict to show up:

(1) MCS  is running as a background task under windows 386.

(2) And MCS is operating as a batch interpreter (running a batch

process) that puts the system into Acquire mode.

What happens is that EGA graphics code that is executed when

MCS  goes  into  Acquire  mode  causes  Windows  to  suspend

execution of the MCS program!

Here is what Windows 386 IS Good for:

(1) turn acquire  on while MCS  is a foreground task

(2) exit to the  MS-DOS Executive

(3) shell COMMAND.COM

Now,  you can  acquire MCS  data totally  in the background,

with  about  600  KB  of  free  ram  availible  to run other

programs.  MCS will  be running  in it's  own 640KB page and

other programs will use in their own 640KB pages. To do this

will require a 386 type AT computer with 2MB or more of RAM.

The file MCS.PIF is provided to run MCS under Windows 386.

4. New Command Line Switches.

'y' Switch:

The  'y' command  line switch  is used  to overcome  a bug some

mouse software drivers that cause  the vertical movement of the

mouse arrow to be upside down.

'g' Switch:

The 'g' command  line switch is used to allow  MCS-II to use the existing

Graphics  Print Screen  Feature. With   this MCS-II  can use  third party

screen  dump utility  programs that  directly support  laser printers and

plotters.

5.   Keyspeed Utility Program

A small utility program called:  KEYSPEED.EXE has been included

to allow users of recent model 80286 and 80386 AT's to speed up

their keyboards. If your computer's ROM BIOS does support extra

fast keyboard repeat rates, the PCA-II channel cursor will move

about four times faster than 'normal'.

If  your  computer's  ROM  BIOS  does  not  support  extra fast

keyboard repeat  rates, the keyboard response  will be unchanged

and no harm will be done.

Usage: keyspeed <speed>

where: speed = 'n' is the normal speed

speed = 'f' is the fastest speed

6. PrintA Utility Program

What  Printa does  is read  in a  single column  ascii spectrum

created  with MCS-II  software, and  write out  a multi  column

ascii spectrum file.

Output  can  go  to  disk  files,  the  computer  screen or the

printer.

PrintA will prompt for the following:

a. Input File.

For proper results the input  file must be an ascii spectrum

file created with some kind of PCA-II software.

b. Output File.

to send output to a disk file, enter any valid DOS filename.

to send output to the CRT enter 'stdout'

to send output to printer enter 'prn'

c. Number of Columns per Row.

This tells the program how many channels to put on each line

of output. Any number between 2 and 32 is ok.

d. Output the Header.

enter 'y'  to include the entire  ascii spectrum file header

in the output

e. Output the ROI data.

enter 'y' to include the ROI data in the output

7. Packing List.

File Name      Purpose

-------------------------------------------------------------------------------------------------------------------

asctobin.c      C language program to convert ascii files to spectrum files

browse.pif      Program Information File for browse.exe (for use with Microsoft Windows)

cuser.c         C Language Source Code for User feature demo program

compile.exe     Batch File Compiler

demo.nbe        Demo Executable Batch Program

demo.nbs        Demo Batch Source File

hw.c            A 'C' program to directly control the mcs card

keyspeed.exe    a program to alter the keyboard speed

nte.exe         Batch File Editor

nteinst.exe     Install Program for the Batch File Editor

mcs.exe         The main MCS program

mcs.pif         Program Information File for mcs.exe (for use with Microsoft Windows)

mcsedit.bat     DOS Batch File to specify what program to use to edit batch files

printa.c        'C' source code to printa.exe

printa.com      program to convert a PCA-II ascii spectrum file into a multiple column file

readme.doc      Last minute documentation File for PCA-II

readme.exe      Program to view the Documentation File (or any text file)

setcolor	Program to toggle colors on and off for screen dumps

switch.c        'C' source code for program to calculate MCS-II card address switch settings

switch.exe      A program to calculate MCS-II card address switch settings

tomcs.c         'C' language source code for the program to convert a PCA-II spectrum file to an MCS-II spectrum file

tomcs.exe       Program to convert a PCA-II spectrum file to an MCS-II spectrum file

vga.doc         Documentation for the VGA setup program

vga.exe         A program to setup the configuration of a VGA or EGA video system

8.  MCS-II Card Switch Settings.

There are four I/O address that have proven to  work well with the MCA-II

card. They are:

Port Number    Switch

-------------  ------------------------------

Hex   Decimal  01  02  03  04  05  06  07  08

-------------  ------------------------------

0210h   528    0   1   0   0   0   0   1   0

07f8h  2040    1   1   1   1   1   1   1   1

01e0    480    0   0   1   1   1   1   0   0

03e0    992    0   0   1   1   1   1   1   0

1 = OPEN.   or the rocker is pushed down next to 'OPEN'

0 = CLOSED. or the rocker is pushed down not next to 'OPEN'

How to come up with other addresses:

Start the SWITCH  program, then enter a number at  the prompt. The Switch

program  will then  draw a  simple diagram  of how  you need  to set  the

address switches for the number that you entered.

####