SUBJECT: MCS-II Software and Hewlett Packard LASERJET III

The current version of MCS-II software does not have the ability

to drive a  Hewlett Packard LASERJET printer. The software works

ok for text reports but does not support the LASERJET III if you

try a graphics screen dump. Until such a time as the MCS-II

software is rewritten to provide drivers for different printers,

another method must be found when a customer needs to do a screen

dump while using a LASERJET printer.

A popular answer is to use a software package from JEWELL

TECHNOLOGIES (1-206-285-6860) called GRAFPLUS. GRAFPLUS does not

allow for any editing of the screen but is faster and only costs

about $60. The GRAPHICS option in MS-DOS V5 will allso provide a

nice screen dump. (If you use GRAFPLUS or MS-DOS V5, start the MCS

software with the "g" switch. ie: MCS g).

A problem with using GRAFPLUS or DOS GRAPHICS option is the screen

colors. If you start up with the screen in color and print to a

monochrome printer, the screen dump will try to do gray shades

where the screen has a background color set. Setting the MCS-II

screen to monochrome prior to the screen dump gives a nice printout

but takes an extra minute or two. When done printing you have to

change it back to normal colors which takes even more time. We

have written a Turbo-C user program that makes the color changes

much faster. The program SETCOLOR will set monochrome colors if

the screen is in color, or will set default colors if the screen

is already in monochrome. In other words, the SETCOLOR program

toggles the MCS-II software back and forth between color and

monochrome.

The SETCOLOR program is called from MCS-II by use of the Options

(ALT-O) menu and the Call a User program option.

Following is a batch file (MCA.BAT) that can be used to tie in

the GRAFPLUS program for use with MCS-II.

C:             ; Make sure that we are talking to the hard disk

CD C:\GRAFPLUS ; GRAFPLUS is in the GRAPLUS subdirectory

GRAFLASR =I1   ; Review the GRAFPLUS manual for command options

CD C:\MCSII    ; MCS-II is in the MCSII subdirectory

MCS n h g      ; Review MCS-II manual for command options

CD C:\GRAFPLUS ; GRAFPLUS is in the GRAPLUS subdirectory

RMGRAF         ; Remove GRAFPLUS from memory

CD C:\         ; Go back to the root directory

If you have MS-DOS V5 on your computer, you may use the GRAPHICS

option that is part of DOS instead. Refer to the DOS manual for

the use of GRAPHICS option.

##########