HW.  the MCS-II Hardware Demo Control Program.

November 29, 1990

I. Introduction.

HW  is a  program that  demonstrates how  to directly  control the MCS-II

hardware. The program is written in  Borland Turbo 'C++' version 1.0. The

source code  contains several large sections  of inline assembly language

source  code.  Because  of  this,  Borland  Turbo  Assembler  2.0 is also

required to rebuild the HW program.

The  HW distribution  disk contains  all executable  and all  source code

files for  HW. HW is made  up of 4910 lines  of source code. (HW  is very

similar to the Background Acquire product for PCA cards.)

Always start  HW by using the  'p<port>' command line parameter.  This is

used to  tell the program  the address of  the I/O port  that your MCS-II

card is using.

Example:

HW p1000 <ENTER>

runs the HW program on an MCS card mapped into I/O port 1000 decimal

(03e8 hex).

Revision dated 21-NOV-1991 by Bob House

This enhanced version of the HW program has the added ability to

send commands to the program from the command line. This will

allow controlling the MCS-II card from a DOS batch file. The

following batch file example shows how to set up a MCS-II card

for one milli-second dwell and leave it in acquire for one pass.

When the acquisition is done, the CYCLE.BAT demo will then save a

spectrum file with the file name TESTDAT.SPM. The command

definitions are the same as shown in later in this document.

Note that the commands that require answers have them as separate

fields with the exception of the port address and file name for

saving a spectrum. The answers will be in the same order as you

would enter them interactively.

CYCLE.BAT

hw p528 r 0 2 0 9 8 7 e 1 m a 1 0 0 5 x

@ECHO OFF

:LOOP

hw r x

CLS

IF ERRORLEVEL 1 GOTO ONE

IF ERRORLEVEL 0 GOTO ZERO

ECHO None of the above...

GOTO NOTDONE

:ONE

ECHO IN ACQUIRE !

GOTO LOOP

:ZERO

ECHO NOT IN ACQUIRE !

:DONE

DEL TESTDAT.SPM

HW r bTESTDAT.SPM x

DIR *.SPM

:NOTDONE

Please note that the above example runs the HW.EXE program in

each loop which creates excessive disk activity. This type of

programming will run MUCH better from a ramdisk.

The commands ARE case sensitive !

This is not a program with a lot of safeguards. If you make a

wrong command sequence it could cause unpredictable results. For

example, if you use command '9' after command 'a' it will be as

if you had not used command 'a'.

Another use of the command line options could be to start up 2 or

more cards from a single batch command. See the following example

for two cards:

hw p528 r 0 2 0 9 8 7 e 1 m 5 x

hw p1000 r 0 2 0 9 8 7 e 1 m 5 x

NOTE: When setting up a MCS-II card for a different port address,

older versions of the SWITCH.EXE program had reversed switch

numbers. This distribution has the corrected version.

1. The Features of HW.

a. Acquire OFF.

This simply takes the MCS-II card out of acquire mode.

b. Test Dwell Counter.

The hard way to set the dwell.

c. Set Group.

This allows users to set the Group size and number.

d. Show Status.

This shows the current system status. This includes:

Acquire. (on/off)

Group.

Preset Pass Count.

Current Pass Count.

The values in the first 4 channels.

Synch.   (internal/external)

Dwell.   (multiplier and scale)

DAC.     (sawtooth/triangle)

e. Read Pass Counter.

Display the values in the current pass counter.

f. Acquire ON.

Puts the MCS-II card into acquire mode.

g. Read RAM.

Display  the counts  in the  first  23  channels on  the text  mode

screen.

h. Zero Address Counter.

Set the hardware channel (read) index to zero.

i. Zero RAM.

Fills the RAM on the MCS-II card with zeros.

j. Zero Pass Counter.

Sets the pass counter to zero.

k. Enter Preset Passes.

Allows the user to enter preset passes as three 16 bit words.

l. Save Spectrum to Disk.

Saves  system  status  and  channel  counts  to  an MCS-II spectrum

(*.SPM) file. Such files can then be read by MCS-II software.

m. Graphics Spectrum View.

This a simple graphics mode  spectrum viewer. It looks and operates

quite a bit like PCA-I software. It is an overlay file that makes a

few calls to the main  HW program using software interrupts. Notice

that the  spectrum viewer (BAV.OVR)  is another program  that calls

code inside HW.EXE by using software interrupt 0f3h, like calling a

DOS of BIOS service from a 'C' program. The spectrum viewer happens

to have been written in Turbo 'C++', but it could just as well have

been written in Turbo Pascal or Modula2.

If this viewer is used while the MCS-II card is in in acquire mode,

the spectrum  will be redrawn about  once a second. That  is if you

have a  pretty fast computer.  The spectrum redisplay  code is very

simple 'radar sweep'. This process involves the following:

(i).   Read the contents of the 8K card into a buffer

(ii).  Scale the data in the buffer for the display

(iii). Update the graphics spectrum display

This  is quite  a lot  of work!  If you  have an  80386SX or faster

computer it works great, otherwise the display will get very 'out of

time'.

If your system has display problems  in acquire mode you can do one

of the following:

(i).   Call show_data() in do_show_data() in BAV1.C less often.

(ii).  Use a block based display refresh method, like what is used

in MCS-II  software. This breaks the  BIG refresh task down

into say, 512 small tasks.

(iii). Get an 80386 system.

n. Toggle the DAC: from/to Sawtooth/Triangle.

o. Enter Dwell Time.

Allows the  scale (ie: 'e' =  External, 'u' = usec,  'm' = msec,...

'D' = day) and a multiplier to be entered.

p. Toggle Internal/External Synch.

q. Make Memory Resident.

Do a soft exit to DOS. When HW is memory resident, it uses up 72704

bytes of system RAM. Of those 72704 bytes, 32768 bytes are for

the program's channel buffer.

r. Exit to DOS.

This does a simple exit to DOS after first updating the config file

'HW.CFG'.  The program  does not  first turn  off acquire  mode. To

restart the program to return to an acquiring MCS-II card start the

program  with   the  'r'  command  line   parameter.  Without  this

parameter, acquire  mode will be  turned off as  the program starts

up.

Examples: (assume that the MCS-II card is mapped into I/O port 1000

decimal)

(i). Normal startup:

HW p1000 <ENTER>

(ii). Return to HW while the MCS-II card is in Acquire mode:

HW p1000 r <ENTER>

HW is  NOT a generic 'driver'  program that has subrountines  that can be

called  by other  programs (because  HW comes  with souce  code, you  can

convert HW into such a driver). HW is not a 'driver' program because most

MCS-II  card  owners  want  to  directly  integrate  'C'  and/or assembly

language code into their 'C' or 'C++' language application programs.

The  feature list  for HW  is just  a subset  of the  features of  MCS-II

software. There is one major difference,  HW comes with 100% source code,

while the source code of MCS-II software is not for sale.

II.  The Files on the HW Disk.

FileName     Purpose

������������������������������������������������������������������������

bav.c        The main source file for the spectrum view module

bav.df       Macros for the spectrum view module

bav.fnt      Text fonts for the spectrum view module

bav.glb      Global data for the spectrum view module

bav.mak      The MAK file for the spectrum view module

bav.ovr      The executable spectrum view module

bav.pro      ANSI prototypes for spectrum view module

bav.str      Text strings for spectrum view module

bav.typ      Data types for spectrum view module

bav1.c       Acquire mode related code for spectrum view module

cycle.bat    Sample batch file to check for acquire status

getscode.asm Assembly language source for function getscode()

getscode.obj Object code for function getscode()

hw.c         Main source code file for HW

hw.cfg       The config file written by HW

hw.doc       This document

hw.exe       The executable HW program

hw.mak       The MAK file for HW

hw_old.c     Older version of source code

hw.typ       Data types for HW

hw0.c        The File I/O source file for HW

one_pass.bat Sample batch file to start two cards

read.exe     A text file viewer to view HW.DOC

userisr.asm  Assembly language source for user_isr()

userisr.obj  Object code for user_isr()

III. Changing HW.

Edit  the source  code with  your programmer's  editor, then rebuild

using MAKE.EXE with the following MAK files: HW.MAK and BAV.MAK. You

will probably need to change the subdirectory names in the MAK files

to match your system.

IV.  Debugging HW.

1. Debugging the Main Program (HW.EXE).

Debug  this program  as you  would any  other 'C'  program, using

Turbo Debugger 2.0 or PeriScope Debugger.

2. Debugging the Spectrum Viewer (BAV.OVR).

Start up HW.EXE, press 'g'  to 'Make Memory Resident', then debug

this program (BAV.EXE) as you  would any other 'C' program, using

Turbo Debugger 2.0  or PeriScope Debugger. The best  way to debug

BAV is  to use thew PeriScope  Debugger on a dual  display system

(Color VGA and a Monochrome TTL).

V.   Converting HW to Microsoft 'C' 6.0

Because 'C' 6.0 now supports inline assembly source code, conversion

of this  code from Turbo 'C++'  1.0 to Microsoft 'C'  6.0 is not too

difficult. Aside from the '\\'  comment delimiters (supported by 'C'

6.0) no real 'C++' features were used in this body of source code.

Do not try  to port this code to any  compiler that does not support

inline  assembly language  source  (ie:  Microsoft 'C'  5.x, Zortech

'C++',  TopSpeed 'C'  or Watcom   'C') unless  you have  an enormous

amount of time to invest in such a project.

Translation of  this code to other  programming languages (ie: Turbo

Pascal or Modula2) would also be a major software project.

What will cause problems are the following:

1. Use of pseudo-variables (ie: _AX, _BX, _CX ...)

convert such code to inline assembly language.

2. Use of the geninterrupt macro

convert such code to inline assembly language.

3. Use of the MK_FP macro

Build initialize pointer variables the 'hard way'.

4. Use of the outportb macro

Replace with outp.

5. Use of the inportb macro

Replace with inp.

VI.  Hardware and Software Requirements.

1. The minimum hardware/software to use HW  includes the following:

a. PC-XT computer

b. EGA video system

c. 72KB of free RAM

d. a disk drive

e. an Oxford Tennelec/Nucleus Inc. MCS-II Board.

f. DOS

2.  The  minimum  hardware/software  to  customize  the  HW code

includes the following:

a. PC-AT

b. EGA video system

c. 570KB of free RAM

d. a hard disk drive

e. an Oxford Tennelec/Nucleus Inc. MCS-II Board.

f. DOS

g. Borland Turbo 'C++' Professional

3. The minimum hardware/software to convert the HW code to a Windows

3.xx application includes the following:

a. a 386-33mhz computer

b. VGA video system

c. 8MB of RAM

d. a Large hard disk drive

e. an Oxford Tennelec/Nucleus Inc. MCS-II Board.

f. a Monochrome TTL video system (for the debugger)

g. DOS

h. Microsoft Windows 3.xx

i. Microsoft Windows 3.xx SDK

j. Microsoft 'C' 6.xx

VII.  How to Operate HW.

The following box shows what the HW main screen looks like.

�������������������������������������������������������������Ŀ

�                                                             �

�      Tennelec/Nucleus Inc. MCS-II Card Controller Demo.     �

�                                                             �

�      0   Acquire OFF.                                       �

�      1   Test Dwell Counter.                                �

�      2   Set Group.                                         �

�      3   Show Status.                                       �

�      4   Read Pass Counter.                                 �

�      5   Acquire ON.                                        �

�      6   Read RAM.                                          �

�      7   Zero Address Counter.                              �

�      8   Zero RAM.                                          �

�      9   Zero Pass Counter.                                 �

�      a   Enter Preset Passes.                               �

�      b   Save Spectrum to Disk.                             �

�      c   Graphics Spectrum View.                            �

�      d   Toggle DAC: Triangle/Sawtooth.                     �

�      e   Enter Dwell Time.                                  �

�      f   Toggle Internal/External Synch.                    �

�      g   Make Memory Resident.                              �

�      x   Exit Program.                                      �

�                                                             �

�������������������������������������������������������������Ĵ

� Last Command: Acquire ON.                                   �

���������������������������������������������������������������

To invoke a command, simply press  the letter just left of the command

name. Letters in single quotes will indicate the pressing of a command

key. Text in double quotes will indicate data entered as a response to

a  prompt.  The  '\\'  are  to  be  considered  as  a  kind of comment

delimiter. Assume that the MCS-II card is mapped into I/O port address

528 decimal.

1. A Standard Run.

Start the HW program.

HW p528 <ENTER>

'0' '9' '8' '7'    // set system to a known state

'2' "0"            // set to full group

'e' "2" "u"        // set dwell to 2 usec

'a' "10" "0" "0"   // set preset passes to 10

'5'                // turn acquire on

'c'                // view the spectrum

'b' "test.spm"     // save the spectrum to a file called TEST.SPM

'x'                // exit to DOS

2. A Long Run.

Start the HW program.

HW p528 <ENTER>

'0' '9' '8' '7'    // set system to a known state

'2' "0"            // set to full group

'e' "1" "S"        // set dwell to 1 Second

'a' "10" "0" "0"   // set preset passes to 10 (a 22.76 hour RUN!)

'5'                // turn acquire on

'c'                // view the spectrum for a moment

'x'                // exit to DOS

Now, leave  the computer on, and  use it for any  other purpose that

does not use the MCS-II card. Notice that no software needs to be in

system  RAM  for  the  data  acquistion  to  continue. The next day,

restart  the HW  program with   the following  command from  the DOS

prompt:

HW p528 r <ENTER>

'c'                // view the complete spectrum

'b' "test.spm"     // save the spectrum to a file called TEST.SPM

'x'                // exit to DOS

#############################################################################